##### MATERIAL SAFETY DATA SHEET

PRODUCTS IDENTIFICATION	:		HARDWOOD PLYWOOD (CARB PHASE II / TSCA TITLE VI)

SYNONYMS:					NONE

TRADE NAME:					BENDING / FLEXPLY PLYWOOD

DESCRIPTION:				THIS PANEL PRODUCT CONTAINS A HARDWOOD

VENEER FACE BONDED TO WOOD COMPONENTS

SUCH AS OTHER WOOD VENEER LUMBER OR

VENEER STRIPS USING UREA-FORMALDEHYDE RESIN.

# POTENTIAL AIRBORNE RELEASES:	THIS PRODUCT MAY

# RELEASE NO MORE THAN 0.05PPM QUANITIES OF

# FORMALDEHYDE EMISSIONS PER THE CARB PHASE II

# REQUIREMENTS.   TPC 6 CERTIFIED.  TSCA Title VI rules.

POTENTIAL AIRBORNE RELEASES:	THIS PRODUCT MAY RELEASE NO MORE THAN  0.05PPM QUANITIES OF FORMALDEHYDE EMISSIONS PER THE CARB PHASE II REQUIREMENTS.   TPC 6 CERTIFIED.

MANUAL OR MECHANICAL CUTTING OR ABRASION PROCESSES

PERFORMED ON THE PRODUCTS CAN RESULT IN GENERATION

OF DUST.

PHYSICAL DATA:

BOILING POINT:				NOT APPLICABLE

SPECIFIC GRAVITY(H20-1):			<1

VAPOR DENSITY:				NOT APPLICABLE

% VOLATIES BY VOL:			0

MELTING POINT:				NOT APPLICABLE

VAPOR PRESSURE:				NOT APPLICABLE

SOLUBILITY IN H20(% BY WGT):		<0.1%

EVAPORATION RATE (BUTYL ACCIATE-1):   NOT APPLICABLE

PH:						NOT APPLICABLE

APPEARANCE & ODOR:			LIGHT TO MEDIUM COLOR. COLOR & ODOR ARE DEPENDENT

UPON WOOD SPECIES.

THIS FACT SHEET IS FOR PRODUCTS THAT HAVE NOT BEEN FINISHED (COATED OR LAMINATED, OR OVERLAID OR TREATED. (FOR EXAMPLE, WITH PRESERVATIVE OF FIRE RETARDENT)

FIRE & EXPLOSION DATA:

FLASH POINT:					NOT APPLICABLE

AUTO IGNITION TEMPERATURE;		NOT AVAILABLE(WILL DEPEND UPON DURATION OF

EXPOSURE TO HEAT SOURCE & OTHER VARIABLES)

EXPLOSIVE LIMITS IN AIR:			SEE BELOW UNDER “UNUSUSAL FIRE AND EXPLOSION

HAZARDS”

EXTINGUISHING MEDIA:			WATER CARBON DIOXIDE & SAND

SPECIAL FIRE FIGHTING PROCEDURES:	NONE

###### UNUSUAL FIRE & EXPLOSION HAZARDS:	SAWING, SANDING OR MACHINING CAN PROSUCE WOOD DUST

AS A BY PRODUCT, WHICH MAY PRESENT AN EXPLOSION

HAZARD IF A DUST CLOUD CONTACTS AN IGNITION SOURCE.

AN AIRBORNE CONCENTRATION OF 40 GRAMS OF DUST PER CUBIC METER OF AIR IS OFTEN USED AS THE L.E.L. FOR WOOD DUST.

REACTIVITY DATA:

CONDITIONS CONTRIBUTING TO

INSTABILITY:					STABLE UNDER NORMAL CONDITIONS

INCOMPATIBILIITY:				AVOID CONTACT WITH OXIDIZING  AGENTS. AVOID OPEN

FLAME.   PRODUCT MAY IGNITE IN EXCESS OF 400F.

HAZARDOUS DECOMPOSITION PRODUCTS:	 THERMAL AND/OR THERMAL OXIDATIVE DECOMPOSITION

CAN PRODUCE IRRITATING AND TOXIC FUMES & GASES,

INCLUDING CARBON MONOXIDE, HYDROGEN, CYANIDE,

ALDEHYDES, ORGANIC ACIDS & POLYNUCLEAR AROMATIC

COMPOUNDS.

HAZARDOUS POLYMERIZATION:		NOT APPLICABLE

HEALTH EFFECTS INFORMATION:

EXPOSURE LIMITS:

FORMALDEHYDE:				CARB PHASE II	 0.05 PPM

EPA Title VI

WOOD DUST (ALL SOFT & HARD		OSHA PEL-TWA	    5 MG/M3

WOODS EXCEPT WEST. RED CEDAR)	OSHA PEL-STEL	  10 MG/M3

HEALTH EFFECTS INFORMATION (CONTINUED)

EYE CONTACT:				GASEOUS FORMALDEHYDE MAY CAUSE TEMPORARY

IRRITATION OR A BURNING SENSATION. WOOD DUST CAN

CAUSE MECHANAICAL IRRITATION.

SKIN CONTACT:				BOTH FORMALDEHYDE & VARIOUS SPECIES OF WOOD DUST

MAY EVOKE ALLERGIC CONTACT DERMATITS INSENSITIZED

INDIVIDUALS.

INGESTION:					NOT LIKELY TO OCCUR.

INHALATION:					GASEOUS FORMALDEHYDE:	MAY CAUSE TEMPORARY

IRRITATION TO EYES, NOSE THROAT. SOME REPORTS SUGGEST THAT FORMALDEHYDE MAY CAUSE RESPIRATORY SENSITATION SUCH AS ASTHMA, AND THAT PER EXISTING RESPIRATORY DISORDERS MAY BE AGGRIVATED BY EXPOSURE.

FORMALDEHYDE IS LISTED BY INTERNATIONAL AGENCY FOR

RESEARCH ON CANCER(LACR) AS A PORBABLE HUMAN

CARCINOGEN THE NATIONAL TOXICOLOGY PROGRAM(NTF)

INCLUDES FORMALDEHYDE IN THE ANNUAL REPORT ON

CARCINOGENS. FORMALDEHYDE IS REGULATED BY OSHA AS A

POTENTIAL CANCER AGENT.

IN STUDIES INVOLVING RATS, FORMALDEHYDE HAD BEEN

SHOWN TO CAUSE NASAL CANCER AFTER  LONG TERM

EXPOSURE TO VERY HIGH CONCENTRATIONS (14+PPM), FAR

ABOVE THOSE NORMALLY FOUND IN THE WORKPLACE USING THIS PRODUCT.

THE NATIONAL CANCER INSTITUTE(NCI) CONDUCTED AN

EPIDEMIOLOGICAL STUDY OF INDUSTRIAL WORKERS EXPOSED

TO FORMALDEHYDE (PUBLISHED JUNE 1986). THE NCI

CONCLUDED THAT THE DATA PROVIDES LITTLE EVIDENCE

THAT MORTALITI FROM CANCER IS ASSOCIATED WITH FORMALDEHYDE EXPOSURE AT THE LEVELS EXPERIENCED BY WORKERS IN THE STUDY.

WOOD DUST:					MAY CAUSE NASAL DRYNESS, IRRATATION AND OBSTRUCTION,

COUGHING, WHEEZING AND SNEEZING, SINUSITIS & PROLONGED COLDS HAVE ALSO BEEN REPORTED.

DEPENDING ON SPECIES MAY CAUSE RESPIRATORY

SENSITIZATION AND/OR IRRITATION. PROLONGED EXPOSURE

TO WOOD DUST HAS BEEN REPORTED BY SOME OBSERVES TO

BE ASSOCIATED WITH NASAL CANCER. WOOD DUST IS NOT	  LISTED AS A CARCINOGEN BY IARC, OR NTP, OR OSHA.

THE POSSIBLE BUILDING OF FORMALDEHYDE GAS

PARTICULARLY WHEN HIGH TEMPERATURES OCCUR.

PRECAUTIONS, SAFE HANDLING:

FORMALDEHYDE:				PROVIDE ADEQUATE VENTILATION TO REDUCE THE

POSSIBLE BUILDING OF FORMALDEHYDE GAS,

PARTICULARLY WHEN HIGH TEMPERATURES OCCUR.

WOOD DUST:					AVOID DUSTY CONDITIONS AND PROVIDE GOOD

VENTILATION.

GENERALLY APPLICABLE CONTROL MEASURES:

VENTILATION:				PROVIDE ADEQUATE GENERAL AND LOCAL EXHAUST

VENTILATION TO KEEP AIRBORNE CONTAMINANT

CONCENTRATION LEVELS BELOW THE OSHA PEL.

PERSONAL PROTECTIVE EQUIPMENT:	WEAR GOGGLES OR SAFETY GLASSES WHEN MANUFACTURING

OR MACHINING THE PRODUCT. WEAR NOSH/MSHA APPROVED

RESPIRATOR WHEN THE ALLOWABLE EXPOSURE LIMITS MAY

BE EXCEEDED. OTHER PROTECTIVE EQUIPMENT SUCH AS

GLOVES AND OUTER GARMENTS MAY BE NEEDED DEPENDING

ON DUST CONDITIONS.

EMERGENCY & FIRST AID PROCEDURES:

EYES:						FLUSH EYES WITH LARGE AMOUNTS OF WATER REMOVE TO

FRESH AIR.	IF IRRITATION PERSISTS, GET MEDICAL

ATTENTION.

SKIN:						WASH AFFECTED AREAS WITH SOAP AND WATER. GET

MEDICAL ADVISE IF RASH OR PERSISTENT IRRITATION OR

DERMATITIS OCCURS.

INHALATION:					REMOVE TO FRESH AIR, GET MEDICAL ADVICE IF PERSISTENT

IRRITATION,	SEVERE COUGHING OR BREATHING DIFFICULTY

OCCURS.

INGESTION:					NOT APPLICABLE

IMPORTER   NAME & ADDRESS                      DATE      SUPERSEDED DATA        PREPARED BY

PANOPLY WOOD PRODUCTS, INC	        07-01-2022                  N/A                         STAFF

414 FIRST STREET

## SOLVANG, CA 93463

IMPORTANT:  THE INFORMATION AND DATA HEREIN ARE BELIEVED TO BE ACCURATE

AND HAVE BEEN COMPILED FROM SOURCES BELIEVED TO BE RELIABLE. IT IS OFFERED

PANOPLY WOOD FOR YOUR CONSIDERATION, INVESTIGATION AND VERIFICATION.

MAKES NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED CONCERNING THE

ACCURACY OR COMPLETENESS OF THE INFORMATION AND DATA HEREIN.  PANOPLY WOOD WILL NOT BE LIABLE FOR CLAIMS RELATING TO ANY PARTYS USE OFF OR RELIANCE ON INFORMATION AND DATA CONTAINED HEREIN REGARDLESS OD WHETHER IT IS CLAIMED THAT THE INFORMATIONAND DATA ARE INACCURATE, INCOMPLETE OR OTHERWISE MISLEADING. IT IS INCUMBENT UPON THE USER TO OBTAIN THE MOST UP TO DATE INFORMATION.